| #include <TMC2209.h> // This code uses the TMC2209 library by Peter Polidoro // https://github.com/janelia-arduino/TMC2209/blob/main/README.md // This code will not work on Arduino boards without HardwareSerial ports, // such as the Uno, Nano, and Mini. // Connect the TMC2209 respective UART pins to the TX1 and RX1 headers HardwareSerial & serial_stream = Serial1; // This variable controls the number of microsteps in the form of 2^MICROSTEPS const uint8_t MICROSTEPS = 4; // This variable ‘VELOCITY’ is the number of microsteps per period const int32_t VELOCITY = 6; // This variable ‘T’ is the time between microsteps in microseconds int T; // This variable ‘SWITCHER’ should be set to 1 if the microsteps per period is rounded up or 0 if it is rounded down int8_t SWITCHER = 1; // This variable ‘TIMER’ is repeatedly assigned the time since the start of the infusion long int TIMER; // Instantiate TMC2209 TMC2209 driver; void setup() { driver.setup(serial_stream); driver.setRunCurrent(100); driver.enableCoolStep(); driver.enable(); // Set the number of microsteps per step driver.setMicrostepsPerStepPowerOfTwo(MICROSTEPS); // Start motor movement at VELOCITY driver.moveAtVelocity(VELOCITY); // Retrieve the inter-microstep period in microseconds T = 256 * driver.getInterstepDuration() / 12; // Set timer to start of program TIMER = micros(); } void loop() { // Delete the loop() contents to remove algorithm // Calculate the time elapsed since the last switch uint32_t DIFF = micros() - TIMER; // Alternate the number of microsteps per period by +/- 1 if (DIFF >= T){ |  |
| --- | --- |
| } | if (SWITCHER == 1){SWITCHER = -1;} else if (SWITCHER == -1){SWITCHER = 1;} VELOCITY += SWITCHER; driver.moveAtVelocity(VELOCITY); T = 256 * driver.getInterstepDuration() / 12; TIMER = micros(); } |

Listing 1: Test Code with Algorithm Applied