Supplementary material (Part 1)

1. Barrett Universal II (BUII): this unpublished paraxial ray-tracing approach is accessible in the IOLMaster 700 optical biometer (software versions 1.70 and 1.88, Carl Zeiss Meditec AG, Jena, Germany). This software provides the formula constant (Lens Factor) for many IOL models including Tecnis 1 ZCB00 (LF = 2.10) (Johnson & Johnson Vision, Jacksonville, FL, USA).

2. Haigis: this formula was programmed into Excel according to the author’s recommendations. 1

3. Hoffer Q: this formula was programmed into Excel according to the author’s recommendations and errata. 2

4. Hoffer QST: this modified version (developed by 3 of the authors (KJH, GS, LT)) of the original Hoffer Q formula relies on AI to predict a new ELP correcting factor (T-factor) and on a nonlinear regression model for AL adjustment in eyes  > 26.00 mm (https://hofferqst.com, version 1.5, accessed on January 11, 2026). 3

5. Holladay 1: this formula was programmed into Excel with the author’s corrections and recommendations. 4

6. Cooke K6 (v2024.01): this is a modified vergence formula, developed by David Cooke, MD, that has three distinctions: a) it transforms the AL provided by optical biometry into a sum-of-segments 5 AL equivalent; b) it predicts the effective lens position (ELP) from measurements and calculations of the secondary principal plane of the actual postoperative IOL position; and c) it back-calculates the total corneal power (this is the reason why it is called the “K” formula). Released in January 2024, it only requires three variables: AL, ACD, and Ks, though central corneal thickness (CCT), and lens thickness (LT) are recommended (https://cookeformula.com/, accessed on January 11, 2026).

7. Kane: this unpublished formula is based on theoretical optics and contains both AI and regression components. 6  It uses 6 variables to predict the IOL power, including mandatory AL, K, ACD and gender, while LT and CCT are optional (http://www.iolformula.com, accessed on January 11, 2026).

8. Norrby thin lens paraxial Ray-tracing (hereafter Norrby RT): this is a paraxial ray-tracing method that introduced a thin refracting surface at the equivalent plane of the IOL. The position of the posterior IOL surface is estimated by the regression formula with AL and ACD as variables. The N constant of -1.01 was used to achieve a zero of mean PE. It was programmed into Excel following the recommendations from the original publication. 7

9. Olsen (OLCR): the Olsen formula is based on a thick-lens paraxial ray-tracing method and is available in 2 versions. The first, defined as Olsen (OLCR), corresponds to the formula installed in the EyeSuite software (version i8.0.0.0, Haag-Streit AG, Switzerland) and uses the C constant concept with ACD and LT as the only IOL position predictors. 8

10. Olsen (standalone): this is the second version of the Olsen formula, is available in the PhacoOptics software (version 1.10.100.2029; IOL Innovations Aps, Aarhus, Denmark) and predicts the IOL position from 4 parameters: AL, K, ACD, and LT.

11. Pearl-DGS: this is a thick-lens formula, developed by G. Debellemanière and D. Gatinel et al. 9 It uses the Cooke-modified AL to approximate sum-of-segments AL, an optimized corneal refractive index, and includes AI predictions (http://www.iolsolver.com, accessed on January 11, 2026).

12. SRK/T: this formula was programmed into Excel following the recommendations from the original article and erratum. 10

13. Voytsekhivskyy Regression Function (VRF): this published formula, which was developed by one of the authors (OVV), is a vergence-based thin-lens formula using the optical CACD constant and 4 variables to calculate the IOL power: AL, K, ACD, and the horizontal CD. 11  It was programmed into Excel by the author (OVV).

14. Voytsekhivskyy Regression Function (VRF) with Cooke-modified AL (hereafter VRF CMAL): this modified version of the VRF formula is based on the AL adjustment proposed by Cooke. 12  A version of CMAL for IOLMaster 500 without using LT was implemented to the formula for AL > 21.90 mm, replacing the original AL correction for eyes ≥ 26.50 mm.  For the short eyes with AL ≤ 21.90 mm the original AL correcting factor of 0.20 mm was used. 13,14  It was programmed into Excel by the author (OVV).

15. Voytsekhivskyy ray-tracing (hereafter VRT): this novel formula, developed by the author (OVV), is a paraxial ray-tracing method based on the thin lens assumption that uses empirically derived ELP algorithm adopted from the VRF formula and Cooke-modified AL to approximate sum-of-segments AL. A version of CMAL that uses LT was implemented in the formula. 5  It uses two constants: CACD and Fc constants, and operates 5 variables, including AL, K, ACD, LT, and horizontal CD. It was programmed into Excel by the author (OVV).

References (supplementary material part 1)

- Haigis W, Lege B, Miller N, Schneider B. Comparison of immersion ultrasound biometry and partial coherence interferometry for intraocular lens calculation according to Haigis. Graefes Arch Clin Exp Ophthalmol. 2000;238(9):765-773. doi:10.1007/s004170000188.

- Hoffer KJ. The Hoffer Q formula: a comparison of theoretic and regression formulas. J Cataract Refract Surg 1993;19(6):700-712; errata 1994: 20:677 and 2007;33(1):2-3. doi:10.1016/s0886-3350(13)80338-0, doi:10.1016/j.jcrs.2006.08.054.

- Hoffer KJ. Hoffer formulas. In: Aramberri J, Hoffer KJ, Olsen T, Savini G, Shammas HJ, eds. Intraocular lens calculations. Essentials in Ophthalmology Springer Cham; 2024:649-660. doi:10.1007/978-3-031-50666-6_43.

- Holladay JT, Prager TC, Chandler TY, Musgrove KH, Lewis JW, Ruiz RS. A three-part system for refining intraocular lens power calculations. J Cataract Refract Surg. 1988;14(1):17-24. doi:10.1016/s0886-3350(88)80059-2.

- Cooke DL, Cooke TL. Approximating sum-of-segments axial length from a traditional optical low-coherence reflectometry measurement. J Cataract Refract Surg 2019; 45(3):351-354. doi: 10.1016/j.jcrs.2018.12.026.

- Melles RB, Kane JX, Olsen T, Chang WJ. Update on intraocular lens power calculation formulas. Ophthalmology 2019; 126(9):1334–1335. doi:10.1016/j.ophtha.2019.04.011.

- Norrby S. Norrby formulas for IOL power calculation. In: Aramberri J, Hoffer KJ, Olsen T, Savini G, Shammas HJ, eds. Intraocular lens calculations. Essentials in Ophthalmology Springer Cham; 2024:705-715. doi:10.1007/978-3-031-50666-6_48.

- Olsen T, Hoffmann P. C constant: New concept for ray tracing–assisted intraocular lens power calculation. J Cataract Refract Surg 2014; 40(5):764-773. doi: 10.1016/j.jcrs.2013.10.037.

- Debellemanière G, Dubois M, Gauvin M, et al. The PEARL-DGS Formula: The development of an open-source machine learning-based thick iol calculation formula. Am J Ophthalmol. 2021;232:58-69. doi:10.1016/j.ajo.2021.05.004.

- Retzlaff JA, Sanders DR, Kraff MC. Development of the SRK/T intraocular lens implant power calculation formula. J Cataract Refract Surg 1990; 16(3):333-340; erratum: 1990 Jul;16(4):528. doi:10.1016/s0886-3350(13)80705-5. doi: 10.1016/S0886-3350(13)80820-6.

- Voytsekhivskyy OV. Accuracy of the VRF and VRF-G intraocular lens power calculation formulas using swept-source optical coherence tomography biometry. Clin Ophthalmol. 2023;17:3663-3672. doi: 10.2147/OPTH.S439287.

- Eom Y, Bae SH, Yang SK, Kim DH, Song JS, Cooke DL. Modified intraocular lens power selection method according to biometric subgroups Eom IOL power calculator. Sci Rep. 2024;14(1):4228. Published 2024 Feb 20. doi:10.1038/s41598-024-54346-9.

- Voytsekhivskyy OV. Development and Clinical Accuracy of a New Intraocular Lens Power Formula (VRF) Compared to Other Formulas. Am J Ophthalmol 2018; 185:56–67. doi:10.1016/j.ajo.2017.10.020.

- Voytsekhivskyy OV. The VRF and VRF-G formulas. In: Aramberri J, Hoffer KJ, Olsen T, Savini G, Shammas HJ, eds. Intraocular lens calculations. Essentials in Ophthalmology Springer Cham; 2024:783-794. doi:10.1007/978-3-031-50666-6_55.

Supplementary material (Part 2)

Voytsekhivskyy Ray-Tracing (VRT) Formula

1) Geometrical Back Focal Distance (GBFD):

GBFD=ALtru-ELPt+RCF+IOLo ;

where

ALtru - true axial length;

ELPt - teoretically derived estimated lens position;

RCF - retina correction factor;

IOLo - IOL offset;

ALtru = (AL(CMAL)+RT);

where

AL(CMAL) - Cooke modified axial length;

RT – retina thickness; RT = +0.20;

AL(CMAL)=(1.23853+0.95855*ALo-0.05467*LT);

where

ALo - AL optical ;

LT – lens thickness;

ELPt = f (CACD; AL(CMAL); K; ACDpre; LT; CD);

ELPt = (((CACD*0.051)-0.006)*(1.23853+0.95855*ALo-0.05467*LT))+(((CACD*0.019)-0.008) *(337.5/Ra))+(((CACD*0.053)+0.005)*ACDpre)-(((CACD*0.013)-0.003)*CD)-((CACD*0.959)-0.013);

Optical CACD constant=(Optical A-constant*0.62467)-68.82;

ELPt corneal power K = 337.5/Ra;

ACDpre – preoperative anterior chamber depth, mm;

CD – horizontal corneal diameter (HWTW), mm;

RCF = retina correction factor;

Depending on the ALo the RCF was set as:

if ALo ≤ 22.00 mm, RCF = -0.04;

if 22.01 mm ≤ ALo ≤ 24.49 mm, RCF = +0.025 ;

if ALo ≥ 24.5 mm,  RCF = +0.022;

IOLo = IOL offset = 0;

Final equation of GBFD:

GBFD = ((1.23853+0.95855*ALo-0.05467*LT)+0.2)-((((CACD*0.051)-0.006)* (1.23853+0.95855*ALo-0.05467*LT))+(((CACD*0.019)-0.008) *K)+(((CACD*0.053)+0.005)*ACDpre)-(((CACD*0.013)-0.003)*CD)-((CACD*0.959)-0.013))+(if (ALo <=22, -0.04, if (ALo >=24.5, 0.022, 0.025)))+(0);

2) Optical Back Focal Distance (OBFD):

VD – vertex distance = 12 mm;

Ray height = 2.5 mm;

Rx – predicted refraction (Ref);

Target Distance (TD) = 6000 mm;

Cornea offset =  0.06 mm;

Corneal power = 331.5/Ra, Ra – corneal anterior curvature;

Fc constant – factor of thick lens, usually 0.2 - 0.45, by default = 0.3;

Refractive index of air  - n1 = 1.000;

Refractive index of aqueous  - n2 = 1.336;

ELPt corneal power K = 337.5/Ra;

Final equation of OBFD:

OBFD = -((((0+(2.5/5988)*5988)+((1*(2.5/5988)-(0+(2.5/5988)*5988)*Rx/1000)/1)*(12-(0.06)))+((1*((1*(2.5/5988)-(0+(2.5/5988)*5988)*Rx/1000)/1)-((0+(2.5/5988)*5988)+((1*(2.5/5988)-(0+(2.5/5988)*5988)*Rx/1000)/1)*(12-(0.06)))*(331.5/Ra)/1000)/1.336)*(0.06+(((-((CACD*0.959)-0.013)+(((CACD*0.051)-0.006)*(1.23853+0.95855*ALo-0.05467*LT))+(((CACD*0.019)-0.008)*337.5/Ra)+(((CACD*0.053)+0.005)*ACDpre)-(((CACD*0.013)-0.003)*CD))))-(0)))+((1.336*((1*((1*(2.5/5988)-(0+(2.5/5988)*5988)*Rx/1000)/1)-((0+(2.5/5988)*5988)+((1*(2.5/5988)-(0+(2.5/5988)*5988)*Rx/1000)/1)*(12-(0.06)))*(331.5/Ra)/1000)/1.336)-(((0+(2.5/5988)*5988)+((1*(2.5/5988)-(0+(2.5/5988)*5988)*Rx/1000)/1)*(12-(0.06)))+((1*((1*(2.5/5988)-(0+(2.5/5988)*5988)*Rx/1000)/1)-((0+(2.5/5988)*5988)+((1*(2.5/5988)-(0+(2.5/5988)*5988)*Rx/1000)/1)*(12-(0.06)))*(331.5/Ra)/1000)/1.336)*(0.06+(((-((CACD*0.959)-0.013)+(((CACD*0.051)-0.006)*(1.23853+0.95855*ALo-0.05467*LT))+(((CACD*0.019)-0.008)*337.5/Ra)+(((CACD*0.053)+0.005)*ACDpre)-(((CACD*0.013)-0.003)*CD))))-(0)))*IOLpower/1000)/1.336)*0)/((1.336*((1.336*((1*((1*(2.5/5988)-(0+(2.5/5988)*5988)*Rx/1000)/1)-((0+(2.5/5988)*5988)+((1*(2.5/5988)-(0+(2.5/5988)*5988)*Rx/1000)/1)*(12-(0.06)))*(331.5/Ra)/1000)/1.336)-(((0+(2.5/5988)*5988)+((1*(2.5/5988)-(0+(2.5/5988)*5988)*Rx/1000)/1)*(12-(0.06)))+((1*((1*(2.5/5988)-(0+(2.5/5988)*5988)*Rx/1000)/1)-((0+(2.5/5988)*5988)+((1*(2.5/5988)-(0+(2.5/5988)*5988)*Rx/1000)/1)*(12-(0.06)))*(331.5/Ra)/1000)/1.336)*(0.06+(((-((CACD*0.959)-0.013)+(((CACD*0.051)-0.006)*(1.23853+0.95855*ALo-0.05467*LT))+(((CACD*0.019)-0.008)*337.5/Ra)+(((CACD*0.053)+0.005)*ACDpre)-(((CACD*0.013)-0.003)*CD))))-(0)))*IOLpower/1000)/1.336)-((((0+(2.5/5988)*5988)+((1*(2.5/5988)-(0+(2.5/5988)*5988)*Rx/1000)/1)*(12-(0.06)))+((1*((1*(2.5/5988)-(0+(2.5/5988)*5988)*Rx/1000)/1)-((0+(2.5/5988)*5988)+((1*(2.5/5988)-(0+(2.5/5988)*5988)*Rx/1000)/1)*(12-(0.06)))*(331.5/Ra)/1000)/1.336)*(0.06+(((-((CACD*0.959)-0.013)+(((CACD*0.051)-0.006)*(1.23853+0.95855*ALo-0.05467*LT))+(((CACD*0.019)-0.008)*337.5/Ra)+(((CACD*0.053)+0.005)*ACDpre)-(((CACD*0.013)-0.003)*CD))))-(0)))+((1.336*((1*((1*(2.5/5988)-(0+(2.5/5988)*5988)*Rx/1000)/1)-((0+(2.5/5988)*5988)+((1*(2.5/5988)-(0+(2.5/5988)*5988)*Rx/1000)/1)*(12-(0.06)))*(331.5/Ra)/1000)/1.336)-(((0+(2.5/5988)*5988)+((1*(2.5/5988)-(0+(2.5/5988)*5988)*Rx/1000)/1)*(12-(0.06)))+((1*((1*(2.5/5988)-(0+(2.5/5988)*5988)*Rx/1000)/1)-((0+(2.5/5988)*5988)+((1*(2.5/5988)-(0+(2.5/5988)*5988)*Rx/1000)/1)*(12-(0.06)))*(331.5/Ra)/1000)/1.336)*(0.06+(((-((CACD*0.959)-0.013)+(((CACD*0.051)-0.006)*(1.23853+0.95855*ALo-0.05467*LT))+(((CACD*0.019)-0.008)*337.5/Ra)+(((CACD*0.053)+0.005)*ACDpre)-(((CACD*0.013)-0.003)*CD))))))*IOLpower/1000)/1.336)*0)*Fc /1000)/1.336).