Appendix A Standard operating procedure for statistical process control.

Panel A Technical acceptability.

Panel B. Clinical event detection.

Notes: We employed statistical process control to pick up the deviation of all measures from baseline during monitoring time. There were 4 steps programmed on the portable hand-held spirometry & electronic questionnaires.

Step 1: 7-day rolling averages were used as the baseline of daily measurement s. Values recorded on subsequent days were judged acceptable if they lay between pre-defined upper and lower limits assuming normal distribution.

Step 2 (Panel A): The higher cut-off value identified the highest 2.5% of normally distributed values (P=0.975). If the measured value was higher than this, subjects were asked to repeat the maneuver and the lowest of the two measures was accepted.

Step 3 (Panel B): The lower cut-off value identified the lowest 5% of normally distributed values (P=0.050). If the measured value was lower than this, subjects were asked to repeat once. If the repeated value was above the threshold, then that value was accepted. If lower again, a clinical event was marked and sent as an alarm to the research center.

Step 4: Alarms on 2 consecutive days were counted as an exacerbation.

## Appendix B Overall adherence scores for old and new software

| Adherence Score (0-6) | Old Software (%) | New Software (%) | Chi-Squared P-value |
| --- | --- | --- | --- |
| 0 | 592 (42.4%) | 340 (27.9%) | <0.001 |
| 1 | 31 (2.2%) | 0 (0.0%) |  |
| 2 | 2 (0.1%) | 0 (0.0%) |  |
| 3 | 5 (0.4%) | 0 (0.0%) |  |
| 4 | 20 (1.4%) | 0 (0.0%) |  |
| 5 | 47 (3.4%) | 33 (2.7%) |  |
| 6 | 701 (50.1%) | 847 (69.4%) |  |
| Total Days monitored | 1398 | 1220 |  |

Notes: Adherence score was ordinal where 0=0% adherence and 6=100% adherence. Overall, subjects on new software had better overall compliance (higher % of high adherence score) than those on the old software (P<0.0001).