Appendix

Here we discuss the effect of signal-to-noise ratio on parameter extraction in T2 mapping, following an approach used for T1 mapping in [45]. One begins with mono-exponential relaxation, and assume that preprocessing leads to a set of magnitude images at Mregularly spaced echo timesTEm=mΔTE where m=1…M. Ideally, data will decay exponentially, with the peak amplitude S0 depending on the magnetization and receiver sensitivity. However, real data will be corrupted by noise, so that the value at the mth echo time can be written as:

Sm=S0 e-m∆TET2+nm

(A1)

The terms nm are derived from Gaussian noise with standard deviation , but become Rician after conversion to magnitude [43]. However, at high SNR the distribution again approximates to a Gaussian. It can be assumed that these conditions hold for all m. If T2 is small, this assumption requires a high SNR at m=1. To extract parameters, one can compare Eqn. A1 with a mono-exponential model:

Sm'=So' e-m∆TET2'

(A2)

Here S0' and T2' are estimates of S0 and T2. At high SNR, we assume that S0' and T2' are close to the actual values, so they can be written as S0'=S0+δS and T2'=T2+δT2 where δS and δT2are small. Retention of first order terms gives:

Sm=S0γm+ nm

Sm'≈S01+mαβ+δSγm

(A3)

Here α=δT2T2, β=ΔTET2 and γ=exp-β. Now, the sum of the squares of the error is ε2=Σm=1MSm-Sm'2, or:

ε2=Σm=1Mnm-S0mαβγm-δSγm2

(A4)

The maximum likelihood estimate (MLE) is obtained when ∂ε2∂t= ∂ε2∂δS= 0. Introducing the notation Σij= Σm=1M miγjm, the separate equations reduce to:

βS0Σ22 α+ Σ12 δS= Σm=1M mγmnm

βS0Σ12 α+ Σ02 δS= Σm=1M γmnm

(A5)

Equations A5 are easily soluble by elimination to obtain:

α= Σ02 Σm=1M mγmnm- Σ12 Σm=1M γmnmβS0Σ02Σ22-Σ122

(A6)

Since the terms nm are uncorrelated, the expected value of α (the normalized error in T2) may be found as:

α= n Σ02Σ11- Σ12Σ01βS0Σ02Σ22-Σ122

(A7)

Here n is the expected value of n. Since n tends to zero as SNR rises, α must also tend to zero, so the mean of T2 is unbiased at high signal-to-noise ratio.

The standard deviation σt of α can also be obtained in terms of the standard deviation of the noise. Since the SNR is S1σt, where S1=S0 e-∆TET2, the result can be written in terms of the standard deviation σT2N of T2 and the SNR as:

σT2N .  SNRT2=G  with  G=γ√Σ02β√Σ02Σ22-Σ122

(A8)

The function G is simple to compute. For example, the lines in Fig A1 show the theoretical variation of G with T2ΔTE, for values of M between 3 and 9. When T2 is low, the curves are similar; however, larger M is needed to reduce G as T2 rises. For M=9, G is almost constant over a wide range, and has the approximate value 1.5. The signal-to-noise ratio needed for a given spread in T2 values may then be estimated as SNR≈1.5T2σT2N. For example, a SNR of 60 is required at T2=40 to achieve a standard deviation σT2N=1. To verify equipment operation, we have estimated G from the product of σT2NT2 and SNR in experiments with multi-tube phantoms, using a body coil for excitation and signal detection. The discrete points in Fig A1 show results obtained at different slice thickness and hence different SNR; the data do indeed lie on the theoretical curve for M=9.

Tissue is not homogeneous. Assuming a distribution of T2 values with standard deviation σT2Hdue to heterogeneity, one would expect from the law of independent variables the overall standard deviation σT2 to be given by:

σT2=√σT2H2+σT2N2

(A9)

Improvements obtained by increasing SNR will therefore limit when σT2=σT2H.

Analysis of the type above may be extended to the matching of a bi-exponential model to a bi-exponential process with signal amplitudes S01 and S02 and time constants T21 and T22. Assuming estimates S0j'=S0j+δS0j and T2j'=T2j+δT2j for each process, one obtains four simultaneous equations analogous to A5:

β1S01Σ220 α1+β2S02Σ211 α2+Σ120 δS01+ Σ111 δS02= Σm=1M mγ1mnm

β1S01Σ211 α1+β2S02Σ202 α2+Σ111 δS01+ Σ102 δS02= Σm=1M mγ2mnm

β1S01Σ120 α1+β2S02Σ111 α2+Σ020 δS01+ Σ101 δS02= Σm=1M γ1mnm

β1S01Σ111 α1+β2S02Σ102 α2+Σ011 δS01+ Σ002 δS02= Σm=1M γ2mnm

(A10)

Here, αj=δT2jT2j, βj=ΔTET2j, γj=exp-βj and Σijk= Σm=1M miγ1jmγ2km. These equations may again be solved to find the normalized errors αjin each time constant. However, they become ill conditioned as T21 and T22 approach each other, when they tend to two degenerate pairs. As many authors have shown [47, 48], bi-exponential fitting requires much higher SNR than mono-exponential fitting when time constants are similar. Despite this, the results are again unbiased, and the separate deviations σT2Njinversely proportional to SNR.

Figures

- Lines show theoretical variations of the function G that represents the link between the normalised standard deviation of T2 and the SNR with the normalised value of T2. Points show experimental values obtained using a body coil and multi-tube phantoms, for M=9 and the different slice thickness and signal-to-noise values shown.

Figure A1.