SUPPLEMENTAL MATERIAL

To: Functional predictors discriminating asthma–COPD overlap (ACO) from chronic obstructive pulmonary disease (COPD)
Kraemer R. et al.

- Assessment of Airway Dynamics by the Integral Method

The assessment of changes in airway caliber to estimate airway function in human subjects and patients with lung disease is best performed by measuring airway resistance using a whole-body plethysmograph, originally introduced by Dubois et al. 54,55. This technique was further adapted by Jaeger and Otis to reach body temperature, ambient pressure and water saturation (BTPS) conditions. Changes in in the calibre of the large, conducting airways are represented by the recording of specific airway loops (sRaw-loops), consisting of the shift volume (Vpleth) and the tidal flow (V’) plot. The lung volume, and hence the resting level of end-expiratory lung volume (EELV) at functional residual capacity (FRCpleth) is logically altered in the presence of marked peripheral airway obstruction or pulmonary hyperinflation. It follows that plethysmography offers measurements of airway dynamics in close relationship to FRCpleth, and hence the EELV, which has considerable advantages in the detection of functional deficits compared to measurements of simple spirometry, impulse oscillometric (IOS) or interrupter resistance (Rint) measurements.

Until recently, and still used in many plethysmographs, sRaw-loops are approximated routinely by  two point analyses creating a straight line throughout the loop.

sRaw=∆VplethV'∙(Pamb-PH2O)

Absolute lung volume FRCpleth is measured by applying a shutter maneuvre and analysing the occlusion-pressure curve.

FRCpleth=∆VplethPm∙Pamb-PH2O

Finally Raw can be calculated from the ration of sRaw and FRCpleth.

Raw=sRawFRCpleth=∆Vpleth∙PmV'∙∆Vpleth∙Pamb-PH2OPamb-PH2O=PmV'

The term (Pamb-PH2O) reflects the dry air pressure. It follows that Raw is inevitably linked to the value of FRCpleth as a fixed parameter in the equation, and is computed misleadingly low, if a pulmonary hyperinflation is present, and consistently high, if the subject breathes under his natural EELV. Noteworthy is that FRCpleth implicates a shutter occlusion manoeuvre, which is sometimes not tolerated by young children or elderly patients, as well as by patients with severe lung involvement. Moreover, Raw consistently ignores the loop-shaping of the sRaw-loop, especially during quiet breathing, and more importantly, if ventilation inhomogeneities are present.

Although numerous parameters of airway resistances can be calculated from plethysmographic measurements 57, the most promising approach was proposed by Matthys H. and Orth U. 58, defining the so called “effective specific resistance” (sReff) as a ratio of the area of the plethysmographic shift-volume versus tidal volume (VplethdVT), to the area of the flow/volume loop (V'dVT) throughout the entire respiratory cycle. Noteworthy, the integral ∆VplethdVT multiplied by the dry air pressure (Pamb-PH2O) embodies the specific, aerodynamic work of breathing (sWOB) at rest 58.  Only advanced computer technology made it possible to assess the two integrals presented for infants, children 59,60, as well as for adults 58,61. The mathematical background computing sReff, sGeff and sWOB, has been presented previously 32,33, showing the following equations:

sReff= Pamb-PH2O* ∆VplethdVTV'dVT= 1sGeff

where (Pamb-PH2O) is the dry air pressure, the integral VplethdVT the equivalent to the area enclosed by sWOB and the integral V'dVT the equivalent to the area of the flow-volume loop. Figure S1 synoptically represents a print-screen from a plethysmographic measurement of a healthy infants obtained in an infant whole-body plethysmograph. 59 It shows a BTPS-compensated plethysmographic loop, which is the flow dependence (V’) from the shift volume (∆Vpleth), derived from two further loops. The first loop (Figure S1; A) represents the tidal flow-volume curve, and its area as integral ∆V'dVT of the

sReff=Pamb-PH2O* ∆VplethdVT∆V'dVT

Figure S1. Print-screen from a plethysmographic measurement of a healthy infant synoptically representing the interaction of two loops, (A) the tidal flow-volume loop and its area as integral ∆V'dVT, and (B) the plethysmographic shift volume-tidal volume, incorporating the area ∆VplethdVT, and hence the sWOB, from which (C) the BTPS-compensated plethysmographic loop, (shift volume (∆Vpleth) against tidal flow (V’) can be derived, and sReff, be computed.

breathing cycle. The second loop (Figure S1; B) characterises the tidal volume in relation to the plethysmographic shift volume and incorporates the area ∆VplethdVT, and hence the sWOB. The determination of the sWOB is a prerequisite and part of the computation of sReff, sGeff, respecively (Figure S1; C), as soon as the subject breathes regularly at resting end-expiratory lung volume (EELV). Importantly however, any extended breathing manoeuvre prior to spontaneous resting breathing has to be avoided, ensuring airway dynamics are assessed under natural breathing conditions e.g., no panting, no breathing against a shutter, no change in the “volume history” 62-64. Normal spontaneous resting breathing is provided, if the subject breaths naturally and regularly at a constant breathing frequency and constant tidal volume (VT) at EELV, and hence with lowest individual sWOB.

2. Defining predictive Equations of Airway Dynamics
(including confounding parameters influencing the actual breathing pattern and timing)

Predictive equations defining reference values of lung function in infants, children and adults are usually based on algorithms of the subject’s anthropometric measurements, such as age, body weight, body height or a combination of them, as independent variables 73,74. Most of these regressions include linear, but also power- or quadric- function relationships 72. This may be suitable for some predictions of static lung volumes 42,71,73,75, volume-time or flow-volume parameters 42,72,73, and indices of intrapulmonary gas distribution 20-2276,77,135. A specific new approach to describe reference ranges more accurately has been developed by Stanojevic et al 78, describing the relationship between spirometric lung function and height, age respectively within the pediatric age range, allowing a seamless transition to adulthood. An extension of the so called “LMS (lambda, mu, sigma) method” 79 was applied. To our knowledge, however, prediction models using LMS-statistics incorporating age and height of the subjects as independent variables have only been formulated for spirometric parameters 75. There is a relative dearth of normative reference values measuring parameters of airway dynamics transitionally from infancy into adulthood. Doershuk et al 80 first presented data of specific airway resistance (sRaw), showing gender differences in infants and children (sRaw males < sRaw females). Measuring airway resistance throughout the whole respiratory cycle in infants was performed by Beardsmore et al 81 demonstrating a dynamic performance of the respiratory circuit in relation to the breathing pattern and suggesting that expiratory looping of the resistance slope could well be due to small airway closure as previously observed by Matthys in adults 2748.

However, the physiologic approach of Matthys and Orth 58 to evaluate airway dynamics by the integral method putting sWOB (VplethdVT) in relation to the tidal flow – volume loop (∆V'dVT) for sReff, its reciprocal ratio for sGeff respectively, has not yet received scientific attention, although this technology has been implemented since years in each Jaeger plethysmograph. Furthermore, based on previous work 29,81 regarding airway patency, but also based on previous work dealing with multiple breath-out techniques 82, the hypothesis was formed that determinants of airway dynamics, EELV, breathing pattern and timing of breathing should be included into the evaluation of parameter prediction of airway dynamics 34. It turned out, that in the approach of defining airway dynamics (sReff, sGeff resp.), the most important factor is sWOB. Therefore, predicting equations must be defined for sWOB, from which in a second step sReff and sGeff can be defined. By a multidimential approach the following regression could be defined 34:

sWOB = EXP(-.300+.017*gender+.138*ln(age)+.836*ln(FRCpleth)
+.744*ln(VT/FRCpleth)+.387*ln(VT/TI) ± .109083)) (SEE)

*gender: male=0; female=1

sGeff = EXP(.816-.050*gender-.423*ln(sWOB)+.415*ln(FRCpleth)

+.603*ln(VT/FRCpleth)) ± .12781 (SEE)

*gender: male=0; female=1

sReff = EXP(-.816+.050*gender+.423*ln(sWOB)-.415*ln(FRCpleth)
-.603*ln(VT/FRCpleth) ± .12781 (SEE)

*gender: male=0; female=1

3. Linear discriminant analysis
(including 17 potentially discriminating lung function parameters)

The 17 predictors were grouped within 5 parameter categories (spirometry: FEV1, FEV1/FVC, FEF25-75; airway dynamics: sReff, sRtot, sWOB; static lung volumes: TLC, FRCpleth, RV, IC/TLC, FRC/TLC, RV/TLC; trapped gases: VTGTLC, VTGFRC, VTGRV, and CO-diffusion:  DLCO, KCO), potentially suitable to discriminate between the 3 diagnostic classes (asthma, ACO and COPD). From the 17 parameters 12 could be selected within the drop-levels by linear discriminant analysis, synoptically presented in Figure S2 and showing the following calculated values in each step of the selection procedure:

|  | Parameter | Wilks lambda | F-statistics overall | p-value overall | F-statistics diff | p-value diff |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | DLCO | 0.4886032 | 281.02564 | 3.053336e-840 | 281.025639 | 3.053336e-84 |
| 2 | FEV1/FVC | 0.3278789 | 200.03469 | 3.741968e-128 | 131.372016 | 0.000000e+00 |
| 3 | sRtot | 0.2669838 | 166.80249 | 1.297182e-149 | 61.012797 | 0.000000e+00 |
| 4 | sWOB | 0.2436810 | 136.93971 | 6.404126e-158 | 25.532860 | 2.564460e-11 |
| 5 | FEF25-75 | 0.2269760 | 117.15218 | 6.058325e-164 | 19.613894 | 6.027593e-09 |
| 6 | VTGFRC | 0.2141793 | 102.92278 | 1.598207e-168 | 15.892758 | 1.976218e-07 |
| 7 | sReff | 0.2041393 | 92.03610 | 5.412915e-172 | 13.057964 | 2.909371e-06 |
| 8 | RV | 0.1966462 | 83.14742 | 2.755107e-174 | 10.097598 | 4.965985e-05 |
| 9 | TLC | 0.1896632 | 76.18737 | 1.804708e-176 | 9.738379 | 7.023176e-05 |
| 10 | IC/TLC | 0.1872110 | 69.23045 | 4.638256e-176 | 3.457972 | 3.220727e-02 |
| 11 | VTGRV | 0.1857499 | 63.25222 | 4.202608e-175 | 2.072762 | 1.268605e-01 |
| 12 | FRC/TLC | 0.1844740 | 58.22229 | 4.470651e-174 | 1.818993 | 1.632059e-01 |
| FEV1, FRCpleth, RV/TLC VTGTLC and KCO excluded by cross-validated correctness rate |  |  |  |  |  |  |

By stepwise classification, using 10-fold cross-validated correctness rate of the R-method “lda”, based on 540 observations with 17 variables in 3 classes with the stop criterion: improvement less than 5%, and according to the Wilks’s lambda statistics (Λ) on each of the discriminant function FEV1, FRCpleth, RV/TLC, VTGTLC and KCO were excluded from the model, giving a correctness rate of 0.8111.

Accordingly, the final overall statistics including these 12 discriminating lung function parameters, presented with the following results:

Accuracy: 0.8981

95% CI: (0.8695, 0.9223)

No Information Rate: 0.6981

P-Value [Acc > NIR]: < 2.2e-16

Kappa: 0. 767

McNamara’s Test P-Value: 0.0004755

Statistics by Class:

|  | Asthma | ACO | COPD |
| --- | --- | --- | --- |
| Sensitivity | 0.9708 | 0.52113 | 0.8913 |
| Specificity | 0.7607 | 0.97655 | 0.9888 |
| Pos. pred. value | 0.9037 | 0.77083 | 0.9425 |
| Neg. pred. value | 0.9185 | 0.93089 | 0.9779 |
| Prevalence | 0.6981 | 0.13148 | 0.1704 |
| Detection rate | 0.6778 | 0.06852 | 0.1519 |
| Detection prevalence | 0.7500 | 0.08889 | 0.1611 |
| Balanced Accuracy | 0.8658 | 0.74884 | 0.9401 |

REFERENCES

(numbered according to the numbering in the main manuscript)

29. Stocks J, Godfrey S, Beardsmore C, Bar-Yishay E, Castile R. Plethysmographic measurements of lung volume and airway resistance. ERS/ATS Task Force on Standards for Infant Respiratory Function Testing. European Respiratory Society/ American Thoracic Society. Eur Respir J. 2001;17(2):302-12. doi: 10.1183/09031936.01.17203020

32. Kraemer R, Blum A, Schibler A, Ammann RA, Gallati S. Ventilation inhomogeneities in relation to standard lung function in patients with cystic fibrosis. Am J Respir Crit Care Med. 2005;171(4):371-8. doi:10.1164/rccm.200407-948OC

33. Kraemer R, Delosea N, Ballinari P, Gallati S, Crameri R. Effect of allergic bronchopulmonary aspergillosis on lung function in children with cystic fibrosis. Am J Respir Crit Care Med. 2006;174(11):1211-20. doi:10.1164/rccm.200603-423OC

34. Kraemer R, Smith HJ, Matthys H. Normative reference equations of airway dynamics assessed by whole-body plethysmography during spontaneous breathing evaluated in infants, children, and adults. Physiol Rep. 2021;9(17): e15027. doi:10.14814/phy2.15027

48. Matthys H. The assessment of airways obstruction. Bull Physiopathol Respir (Nancy). 1973;9(4):961-78.

54. Dubois AB, Botelho SY, Bedell GN, Marshall R, Comroe JH, Jr. A rapid plethysmographic method for measuring thoracic gas volume: a comparison with a nitrogen washout method for measuring functional residual capacity in normal subjects. J Clin Invest. 1956;35(3):322-6.

55. Dubois AB, Botelho SY, Comroe JH, Jr. A new method for measuring airway resistance in man using a body plethysmograph: values in normal subjects and in patients with respiratory disease. J Clin Invest. 1956;35(3):327-35.

56. Jaeger MJ, Otis AB. Measurement of Airway Resistance with a Volume Displacement Body Plethysmograph. J Appl Physiol. 1964;19:813-20.

57. Ulmer WT, Marek W, Rasche K. [Airway resistance curves in obstructive respiratory tract diseases. 8 types of airway resistance curves in spontaneous respiration]. Fortschr Med. 1988;106(33):663-4, 665-6, 667.

58. Matthys H, Orth U. Comparative measurements of airway resistance. Respiration. 1975;32(2):121-34.

59. Kraemer R. Whole-body plethysmography in the clinical assessment of infants with bronchopulmonary diseases. Respiration. 1993;60(1):1-8.

60. Kusenbach G, Pott H, Breuer A, et al. Evaluation of a commercial infant body plethysmography. Eur Respir J. 1998;11(5):1211-3.

61. Zaiss AW, Matthys H. A multiuser system for whole body plethysmographic measurements and interpretation. Lung. 1990;168 Suppl:1185-92.

62. Agostoni E, Mead, J. Statics of the respiratory system. In: Fenn WO, Rahn, H. , ed. Handbook of Physiology. American Physiological Society, Washington D.C. ed. 1964:411-427:chap 14.

63. Mead J, Turner JM, Macklem PT, Little JB. Significance of the relationship between lung recoil and maximum expiratory flow. J Appl Physiol. 1967;22(1):95-108.

64. Duggan CJ, Castle WD, Berend N. Effects of continuous positive airway pressure breathing on lung volume and distensibility. J Appl Physiol (1985). 1990;68(3):1121-6.

71. Stocks J, Quanjer PH. Reference values for residual volume, functional residual capacity and total lung capacity. ATS Workshop on Lung Volume Measurements. Official Statement of The European Respiratory Society. Eur Respir J. 1995;8(3):492-506.

72. Hankinson JL, Odencrantz JR, Fedan KB. Spirometric reference values from a sample of the general U.S. population. Am J Respir Crit Care Med. 1999;159(1):179-87. doi:10.1164/ajrccm.159.1.9712108

73. Zapletal A, Samanek M, Paul T. Lung function in children and adolescents. Karger; 1987.

74. Quanjer PH, Stocks J, Polgar G, Wise M, Karlberg J, Borsboom G. Compilation of reference values for lung function measurements in children. Eur Respir J Suppl. 1989;4:184S-261S.

75. Quanjer PH, Stanojevic S, Cole TJ, et al. Multi-ethnic reference values for spirometry for the 3-95-yr age range: the global lung function 2012 equations. Eur Respir J. 2012;40(6):1324-43. doi:10.1183/09031936.00080312

76. Kraemer R, Meister B. Fast real-time moment-ratio analysis of multibreath nitrogen washout in children. J Appl Physiol. 1985;59(4):1137-44. doi: 10.1152/jappl.1985.59.4.1137

77. Lum S, Stocks J, Stanojevic S, et al. Age and height dependence of lung clearance index and functional residual capacity. Research Support, Non-U.S. Gov't. Eur Respir J. 2013;41(6):1371-7. doi:10.1183/09031936.00005512

78. Stanojevic S, Wade A, Stocks J, et al. Reference ranges for spirometry across all ages: a new approach. Am J Respir Crit Care Med. 2008;177(3):253-60. doi:200708-1248OC [pii]

10.1164/rccm.200708-1248OC

79. Cole TJ, Stanojevic S, Stocks J, Coates AL, Hankinson JL, Wade AM. Age- and size-related reference ranges: a case study of spirometry through childhood and adulthood. Stat Med. 2009;28(5):880-98. doi:10.1002/sim.3504

80. Doershuk CF, Fisher BJ, Matthews LW. Specific airway resistance from the perinatal period into adulthood. Alterations in childhood pulmonary disease. Am Rev Respir Dis. 1974;109(4):452-7.

81. Beardsmore CS, Godfrey S, Shani N, Maayan C, Bar-Yishay E. Airway resistance measurements throughout the respiratory cycle in infants. Respiration. 1986;49(2):81-93.

82. Yammine S, Singer F, Gustafsson P, Latzin P. Impact of different breathing protocols on multiple-breath washout outcomes in children. J Cyst Fibrosis. 2014;13(2):190-7. doi:10.1016/j.jcf.2013.08.01

Figure legends

Figure S1. Print-screen from a plethysmographic measurement of a healthy infant synoptically representing the interaction of two loops, (A) the tidal flow-volume loop and its area as integral ∆V'dVT, and (B) the plethysmographic shift volume-tidal volume, incorporating the area ∆VplethdVT, and hence the sWOB, from which (C) the BTPS-compensated plethysmographic loop, (shift volume (∆Vpleth) against tidal flow (V’) can be derived, and sReff, be computed.

Figure S2. Predictors potentially suitable to discriminate between the 3 diagnostic classes (asthma, ACO and COPD), assessed by linear discriminant analysis (LDA) using R-statistics.

Figures

The work of breathing loop

sWOB=VplethdVT

The breathing cycle

Tidal flow-tidal volume loop

∆V'dVT

The plethysmographic

tidal flow – shift volume loop

A

B

C

zero-flow

zero-flow

∆Vpleth

Volume - shift volume

Flow(tidal) – Volume(tidal)

sReff: 0.96 kPa*s

∆Vpleth

Figure S1

Figure S2