Supplementary table 1: Inclusion and Exclusion criteria for this study

| condition | content |
| --- | --- |
| Inclusion criteria | 01)It meets the diagnostic criteria for hepatocellular carcinoma in the "Primary Liver Cancer Diagnosis and Treatment Standards 2019 Edition". 02)The maximum diameter of a single tumor is ≤ 5 cm, or the number of tumors is ≤ 3 and the maximum diameter of the tumor is ≤ 3 cm 03)Imaging examination showed no macrovascular invasion, lymphatic metastases, or extrahepatic metastases 04)There is no history of other malignant tumors or serious other organ lesions in the past 05)Preoperative noncontract scan + dynamic contrast MRI examination 06)The patient profile was complete and the follow-up was complete 07)ECOG score ≤2 08)Child-Pugh (CTP) grade A or B 09)Radiofrequency or microwave after the first treatment of the technical assessment successful |
| Exclusion criteria | 01)The number of intrahepatic tumors was greater than 3 02)Accompanied by portal or venous cancer thrombosis or invasion of adjacent organs 03)CTP grade C, which cannot be improved by hepatoprotective therapy 04)Esophageal and gastric variceal bleeding within 4 weeks prior to treatment 05)Uncorrectable coagulation disorders and severe blood abnormalities, severe bleeding tendency 06)Refractory massive ascites, cachexia 07)There is an active infection, especially inflammation of the biliary system 08)Severe failure of major organs such as liver, kidney, heart, lungs, and brain 09)Patients who are unconscious or unable to cooperate with treatment 10) Contrast agents are hepatocyte-specific contrast agents 11)Prior to ablation, any of the following treatments: chemotherapy, targeted, immunotherapy 12)Patients lost to follow-up within 6 months after ablation treatment 13)Those who are not completely abled |

Note: Definition of success by radiofrequency or ablation techniques: Repeat dynamic contrast MRI within 30 days after ablation, indicating that the target tumor is completely covered by the ablation area. Definition of incomplete ablation: the first repeat MRI after ablation, indicating that the target tumor is not completely covered by the ablation area.

Supplementary table 2: MRI equipment parameters in three hospitals

1.Upper abdominal dynamic enhancement MRI scanning equipment and parameters of Sun Yat-sen University Cancer Center

| MR models and sequences | azimuth | TR（ms） | TE（ms） | FOV | Bandwidth (kHz) | thickness (mm) | Layer spacing | matrix | Number of excitations |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GE 3.0 T |  |  |  |  |  |  |  |  |  |
| FSE T2WI | cross section | 10588 | 73 | 380×380 | 83 | 7 | 0.7 | 320×320 | 2 |
| SSFSE T2WI | Coronal plane | 10600 | 100 | 400×400 | 42 | 5 | 0.5 | 288×224 | 2 |
| T1WI pre-scan LAVA | cross section | 3.7 | 1.6 | 400×360 | 167 | 5 | -2 | 260×224 | 1 |
| T1WI Enhanced LAVA | cross section | 2.8 | 1.3 | 400×360 | 167 | 5 | -2 | 260×224 | 1 |
| Siemens 3.0 T |  |  |  |  |  |  |  |  |  |
| FSE T2WI | cross section | 2200 | 87 | 380×380 | 710 | 5 | 1.5 | 320×256 | 2 |
| SSFSE T2WI | Coronal plane | 2200 | 92 | 380×380 | 710 | 6 | 0.6 | 320×225 | 2 |
| T1WI pre-scan LAVA | cross section | 3.9 | 1.89 | 400×360 | 440 | 3 | 0.6 | 288×201 | 1 |
| T1WI Enhanced LAVA | cross section | 3.9 | 1.89 | 400×360 | 440 | 3 | 0.3 | 288×201 | 1 |
| United Imaging 3.0 T |  |  |  |  |  |  |  |  |  |
| T2WI FSE FS Trigger | cross section | 4000 | 78.1 | 380×300 | 330 | 7 | 0.7 | 288×216 | 3 |
| T2WI FSE SPIR Trigger | Coronal plane | 4000 | 82.94 | 360×320 | 355 | 5 | 0.5 | 288×217 | 3 |
| T1WIPre-scan quick 3D | cross section | 2.91 | 1.31 | 400×320 | 800 | 5 | -2 | 256×202 | 1 |
| T1W Iintensifier quick 3D | cross section | 2.91 | 1.31 | 400×360 | 800 | 5 | -2 | 256×203 | 1 |
| Philips 3.0 T |  |  |  |  |  |  |  |  |  |
| T2WI SPIR Trigger | cross section | 2103 | 76 | 400×400 | 1413.2 | 6 | 0.6 | 268×268 | 1 |
| T2WI TSE FS Trigger | Coronal plane | 710 | 80 | 350×350 | 420.9 | 6 | 0.6 | 220×198 | 1 |
| T1WI pre-scan THRIVE | cross section | 3.7 | 1.32 | 300×360 | 1260.6 | 4.5 | -2.5 | 268×236 | 1 |
| T1WI intensifier THRIVE | cross section | 3.7 | 1.32 | 300×360 | 1260.6 | 4.5 | -2.5 | 268×236 | 1 |

2.Upper abdominal artery enhanced MRI scanning equipment and parameters in Henan Provincial Cancer Hospital

| MR models and sequences | azimuth | TR（ms） | TE（ms） | FOV | Bandwidth (kHz) | thickness (mm) | Layer spacing | matrix | Number of excitations |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GE 3.0 T |  |  |  |  |  |  |  |  |  |  |
| FSE T2WI | cross section | 10588 | 73 | 380×380 | 83 | 7 | 0.7 | 320×320 | 2 |  |
| SSFSE T2WI | Coronal plane | 10600 | 100 | 400×400 | 42 | 5 | 0.5 | 288×224 | 2 |  |
| T1WI pre-scan LAVA | cross section | 3.7 | 1.6 | 400×360 | 167 | 5 | -2 | 260×224 | 1 |  |
| T1WI Enhanced LAVA | cross section | 2.8 | 1.3 | 400×360 | 167 | 5 | -2 | 260×224 | 1 |  |
| Siemens 3.0 T |  |  |  |  |  |  |  |  |  |  |
| FSE T2WI | cross section | 2200 | 87 | 380×380 | 710 | 5 | 1.5 | 320×256 | 2 |  |
| SSFSE T2WI | Coronal plane | 2200 | 92 | 380×380 | 710 | 6 | 0.6 | 320×225 | 2 |  |
| T1WI pre-scan LAVA | cross section | 3.9 | 1.89 | 400×360 | 440 | 3 | 0.6 | 288×201 | 1 |  |
| T1WI Enhanced LAVA | cross section | 3.9 | 1.89 | 400×360 | 440 | 3 | 0.3 | 288×201 | 1 |  |
| United Imaging 3.0 T |  |  |  |  |  |  |  |  |  |  |
| T2WI FSE FS Trigger | cross section | 4000 | 78.1 | 380×300 | 330 | 7 | 0.7 | 288×216 | 3 |  |
| T2WI FSE SPIR Trigger | Coronal plane | 4000 | 82.94 | 360×320 | 355 | 5 | 0.5 | 288×217 | 3 |  |
| T1WI PRE-SCAN THRIVE | cross section | 2.91 | 1.31 | 400×320 | 800 | 5 | -2 | 256×202 | 1 |  |
| T1WI INTENSIFIER THRIVE | cross section | 2.91 | 1.31 | 400×360 | 800 | 5 | -2 | 256×203 | 1 |  |

3.Upper abdomen dynamic enhancement MRI scanning equipment and parameters in Huaihe Hospital of Henan University

| MR models and sequences | azimuth | TR（ms） | TE（ms） | FOV | Bandwidth (kHz) | thickness (mm) | Layer spacing | matrix | Number of excitations |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GE 3.0 T |  |  |  |  |  |  |  |  |  |
| FSE T2WI | cross section | 10588 | 73 | 380×380 | 83 | 7 | 0.7 | 320×320 | 2 |
| SSFSE T2WI | Coronal plane | 10600 | 100 | 400×400 | 42 | 5 | 0.5 | 288×224 | 2 |
| T1WI pre-scan LAVA | cross section | 3.7 | 1.6 | 400×360 | 167 | 5 | -2 | 260×224 | 1 |
| T1WI Enhanced LAVA | cross section | 2.8 | 1.3 | 400×360 | 167 | 5 | -2 | 260×224 | 1 |
| Siemens 3.0 T |  |  |  |  |  |  |  |  |  |
| FSE T2WI | cross section | 2200 | 87 | 380×380 | 710 | 5 | 1.5 | 320×256 | 2 |
| SSFSE T2WI | Coronal plane | 2200 | 92 | 380×380 | 710 | 6 | 0.6 | 320×225 | 2 |
| T1WI pre-scan LAVA | cross section | 3.9 | 1.89 | 400×360 | 440 | 3 | 0.6 | 288×201 | 1 |
| T1WI Enhanced LAVA | cross section | 3.9 | 1.89 | 400×360 | 440 | 3 | 0.3 | 288×201 | 1 |

Supplementary table 3: Ablation parameters for three hospitals

| The name of the center | equipment | power | frequency |
| --- | --- | --- | --- |
| Sun Yat-sen University Cancer Center | Nanjing-Yigao (Microwave) | 50-60W | 2450MHz |
|  | Nanjing-Minimally Invasive (Microwave) | 50-60W | 2450MHz |
|  | Nanjing-The Great Wall (Microwave) | 50-60W | 2450MHz |
|  | Cool-tip (RF) | 100-120W | 480kHz |
| Henan Provincial Cancer Hospital | Nanjing Yigao (microwave) | 50-60W | 2450MHz |
|  | Mianyang-Lide (RF) | 90-120W | 400kHz |
|  | Boston Scientific (RF) | 90-110W | 500kHz |
| Huaihe Hospital of Henan University | Nanjing Yigao(microwave). | 50-60W | 2450MHz |
|  | Boston Scientific (RF) | 90-110W | 500kHz |

Supplementary Table 4: The selected features and their coefficients in different ROI of phases in the training set

| Series | Features | Coefficient values |
| --- | --- | --- |
| AC | log.sigma.1.0.mm.3D_glcm_InverseVariancer wavelet.HLL_glcm_MCC wavelet.LHL_glrlm_LongRunHighGrayLevelEmphasis | -7.961e-01 7.768e-01 1.596e-05 |
| VC | log.sigma.4.0.mm.3D_glszm_SmallAreaLowGrayLevelEmphasis wavelet.LHH_glszm_ZoneEntropy | 1.22472 0.07919 |
| T2 | log.sigma.3.0.mm.3D_firstorder_Skewness wavelet.LHH_firstorder_RootMeanSquared | -0.15957 0.07256 |
| DC | wavelet.LLH_glszm_SmallAreaLowGrayLevelEmphasis | -0.51527 |
|  | wavelet.HLH_firstorder_Mean | 0.49291 |
|  | wavelet.HHH_glszm_SmallAreaLowGrayLevelEmphasis | 0.42068 |
|  | wavelet.HLL_firstorder_Skewness | -0.10567 |
| FLEX | wavelet.HHH_glcm_Imc1 log.sigma.1.0.mm.3D_firstorder_Skewness wavelet.HHL_firstorder_Mean wavelet.LHH_firstorder_Median | 1.67641 -0.02772 0.29909 0.41206 |

Note: L represents low-pass filter, H represents low-pass filter. For example, HHL represents the intensity value resulting from directional filtering of X with a high-pass filter along x-direction, a high pass filter along y-direction and a low-pass filter along z-direction.

Supplementary Data 1: Five-Sequence Integrated Radiomics Score

Radiomics score =

(0.4225

+ (-0.7961) * log(sigma.1.0.mm.3D_glcm_InverseVariance)

+ 0.7768 * wavelet.HLL_glcm_MCC

+ 0.00001596 * wavelet.LHL_glrlm_LongRunHighGrayLevelEmphasis)

+ (0.55137

+ 1.67641 * wavelet.HHH_glcm_Imc1

+ (-0.02772) * log(sigma.1.0.mm.3D_firstorder_Skewness)

+ 0.29909 * wavelet.HHL_firstorder_Mean

+ 0.41206 * wavelet.LHH_firstorder_Median)

+ (-72.08426

+ (-0.15957) * log(sigma.3.0.mm.3D_firstorder_Skewness)

+ 0.07256 * wavelet.LHH_firstorder_RootMeanSquared)

+ (0.27313

+ 1.22472 * log(sigma.4.0.mm.3D_glszm_SmallAreaLowGrayLevelEmphasis)

+ 0.07919 * wavelet.LHH_glszm_ZoneEntropy)

+ (0.46502

+ (-0.51527) * wavelet.LLH_glszm_SmallAreaLowGrayLevelEmphasis

+ 0.49291 * wavelet.HLH_firstorder_Mean

+ 0.42068 * wavelet.HHH_glszm_SmallAreaLowGrayLevelEmphasis

+ (-0.10567) * wavelet.HLL_firstorder_Skewness)

Supplementary Figure 1：The Process of Manual Segmentation of Five Sequences

This image displays the manual delineation and 3D modeling of a hepatocellular carcinoma tumor from five MRI sequences:

FLEX: MRI plain scan.

AC: MRI arterial phase.

VC: MRI portal venous phase.

DC: MRI delayed phase.

T2: MRI T2-weighted imaging.

Each row shows 2D scans and their corresponding 3D tumor models, used for radiomic feature extraction and deep learning applications.